E0 217 Project Report (Aug 2023)

Names and IISc Email IDs of Group Members:

|  | Name | IISc Email ID |
| --- | --- | --- |
| 1. | Guhan Rajasekar | guhanr@iisc.ac.in |
| 2. | Ujjwal Chaudhary | ujjwalc@iisc.ac.in |

Individual Contributions

|  | Name | Contributions |
| --- | --- | --- |
| 1. | Guhan Rajasekar | - Implementation of FFT code ( without control signals) - Error computation between obtained and desired results - Report compilation |
| 2. | Ujjwal Chaudhary | - Optimization of code ( changing combinational blocks to sequential blocks) and making the code compatible with CLK signal and control signals like RST, WRITE, START , READY - Creation of test bench and simulation of design in GTK Wave. - Performed area, power and speed analysis. |

Number of clock cycles per FFT computation:

- Number of clock cycles between beginning of start and beginning of ready signal = 4.5

Area of synthesized design:

- 13366.234 µm2.

Max clock frequency supported by the synthesized design:

- Typical corner, 25℃ and 1V

- This result is for typical corner, 25℃ and 1V.

- Here the clock period is 5ns – 0.11ns = 4.89ns

- Here the slack period is 0.11ns.

- Operating frequency is 204.49 MHz.

- Slow corner, 25℃ , 1V

- This is for slow corner, 25℃ and 1V.

- Here the time period is 20ns – 0.77ns = 19.23ns

- Operating frequency  = 52.002 MHz.

Power Consumption of the synthesized design:

- Power Consumption in typical corner, 25℃ and 1V:

- The total power consumed at typical corner, 25℃ and 1V is 14.9mW.

- Power Consumption in FF corner,25℃ and 1V.

- Total power consumption at FF corner, 25℃ and 1V is 22.7mW.

Energy Consumption of synthesized design:

| Corner Case | Power Consumption | Operating Frequency | Energy Consumption |
| --- | --- | --- | --- |
| Typical , 25℃ , 1V | 14.9mW | 204.49MHz | 72.86μJ |

Dependence of Energy Consumption of the synthesized design on the clock frequency:

- For a given simulation case, energy consumed is inversely related to the operating frequency.

- As the operating frequency increases, the energy consumed decreases and vice-versa.

Maximum absolute error when computing FFT of {0, 1, 2, 3, 4, 5, 6, 7}

- Let the 8-point FFT result be denoted as X(0), X(1), X(2), X(3), X(4), X(5), X(6) and  X(7)

- Here a decimal point has been added in the obtained results to indicate there are 8 bits for the fractional part, 7 bits for the decimal part and MSB is the sign bit. This holds true for both real and imaginary parts.

- In the calculation of error, 2 is considered as 1.414.

- X(0) :

- Expected                =   28 + j0

- Obtained                =   (1c.00 + j00.00)H = 28 + j0

- Absolute  Error     =   0

- X(1)

- Expected               =   -4.0 + j 9.656

- Obtained               =   (fc.00 + j09.a0)H = -4 + j9.625

- Absolute Error      =  |(-4.0 + j 9.656) – (-4 + j9.625) | = 0.031

- X(2)

- Expected               =   -4 + j4

- Obtained               =  (fc.00 + j04.00)H = (-4 + j4)

- Absolute Error      =  0

- X(3)

- Expected              =    -4 + j1.656

- Obtained              =    (fc.00 + j01.a0)H  = -4 + j1.625

- Absolute Error     =    |(-4 + j1.656) – (-4 + j1.625) | = 0.031

- X(4)

- Expected              =   -4 + j0

- Obtained              =   (fc.00 + j00.00)H  = -4 + j0

- Absolute  Error    =  0

- X(5)

- Expected               =  -4 – j1.656

- Obtained               =  (fc.00 + jfe.60)H = (-4 – j1.402)

- Absolute  Error     =  |(-4 – j1.656  ) – (-4 – j1.402)  |  = 0.254

- X(6)

- Expected                 =   -4.0 – j4.0

- Obtained                 =   (fc.00 + jfc.00)H = -4 - j4

- Absolute  Error       =   0

- X(7)

- Expected                 =  -4 – j9.656

- Obtained                 =  (fc.00 + jf6.60)H = -4 – j9.625

- Absolute  Error       =  | (-4 – j9.656 ) – (-4 – j9.625) | = 0.031

Modifying the fft circuit to compute inverse fft

- Inverse fft can be computed using Data Swapping technique, as shown below:

- The real part of FFT input is fed as imaginary input to the circuit and the imaginary part of FFT is fed as real input to the circuit.

- The real portion of the output gives imaginary part of expected time domain sequence scaled by N  and  the imaginary part of the output gives the real part of the time domain sequence scaled by N.

- To get the desired outputs, we divide them by N.

Design Details: