E0 217 Project Report (Aug 2023)

Names and IISc Email IDs of Group Members:

|  | Name | IISc Email ID |
| --- | --- | --- |
| 1. | Guhan Rajasekar | guhanr@iisc.ac.in |
| 2. | Ujjwal Chaudhary | ujjwalc@iisc.ac.in |

Individual Contributions

|  | Name | Contributions |
| --- | --- | --- |
| 1. | Guhan Rajasekar | - Implementation of FFT code ( without control signals) - Error computation between obtained and desired results - Report compilation |
| 2. | Ujjwal Chaudhary | - Optimization of code ( changing combinational blocks to sequential blocks) and making the code compatible with CLK signal and control signals like RST, WRITE, START , READY - Creation of test bench and simulation of design in GTK Wave. - Performed area, power and speed analysis. |

Number of clock cycles per FFT computation:

- Number of clock cycles between beginning of start and beginning of ready signal = 4.75

Area of synthesized design:

- Area of synthesized design is 13367.298 µm2.

Max clock frequency supported by the synthesized design:

- Typical corner, 25℃ and 1V

- This result is for typical corner, 25℃ and 1V.

- Input clock signal period is 5ns.

- Here the slack period is 0.26ns.

- Supported clock period is  5ns – 0.26ns = 4.74ns

- Operating frequency is 210.97 MHz.

- SS corner, 25℃ , 1V

- This is for SS corner, 25℃ and 1V.

- Input clock period is 20ns. ( because this case does not support clock signal of 5ns)

- Slack period is 0.66 ns.

- Supported clock signal period is 20ns – 0.66ns = 19.34ns

- Operating frequency  = 51.706 MHz.

- FF corner, 25℃ , 1V

- Input clock signal period = 5ns.

- Slack period  = 2.54 ns.

- Clock period supported by the design = 2.46ns.

- Max operating frequency at FF corner = 406.504MHz

Power Consumption of the synthesized design:

- Power Consumption in typical corner, 25℃ and 1V:

- The total power consumed at typical corner, 25℃ and 1V is 15.1mW.

- Power Consumption in SS corner,25℃ and 1V.

- Power consumed at SS corner, 25℃ and 1V is 24.5mW

- Power Consumption in FF corner,25℃ and 1V.

- Total power consumption at FF corner, 25℃ and 1V is 27.2mW.

Energy Consumption of synthesized design:

| Corner Case | Power Consumption (mW) | Operating Frequency (MHz) | Energy Consumption (pJ) |
| --- | --- | --- | --- |
| Typical , 25℃ , 1V | 15.1 | 210.97 | 71.57 |
| FF , 25℃ , 1V | 27.2 | 406.504 | 66.91 |

Dependence of Energy Consumption of the synthesized design on the clock frequency:

- As we go from typical corner to FF corner, operating frequency increases and energy consumption reduces.

- So based on this, we conclude that energy consumption reduces as clock frequency increases.

Maximum absolute error when computing FFT of {0, 1, 2, 3, 4, 5, 6, 7}

- Let the 8-point FFT result be denoted as X(0), X(1), X(2), X(3), X(4), X(5), X(6) and  X(7)

- Here a decimal point has been added in the obtained results to indicate there are 8 bits for the fractional part, 7 bits for the decimal part and MSB is the sign bit. This holds true for both real and imaginary parts.

- In the following calculations, 2 is considered as 1.414.

- X(0) :

- Expected                =   28 + j0

- Obtained                =   (1c.00 + j00.00)H = 28 + j0

- Absolute  Error     =   0

- X(1)

- Expected               =   -4.0 + j 9.656

- Obtained               =   (fc.00 + j09.a0)H = -4 + j9.625

- Absolute Error      =  |(-4.0 + j 9.656) – (-4 + j9.625) | = 0.031

- X(2)

- Expected               =   -4 + j4

- Obtained               =  (fc.00 + j04.00)H = (-4 + j4)

- Absolute Error      =  0

- X(3)

- Expected              =    -4 + j1.656

- Obtained              =    (fc.00 + j01.a0)H  = -4 + j1.625

- Absolute Error     =    |(-4 + j1.656) – (-4 + j1.625) | = 0.031

- X(4)

- Expected              =   -4 + j0

- Obtained              =   (fc.00 + j00.00)H  = -4 + j0

- Absolute  Error    =  0

- X(5)

- Expected               =  -4 – j1.656

- Obtained               =  (fc.00 + jfe.60)H = (-4 – j1.402)

- Absolute  Error     =  |(-4 – j1.656  ) – (-4 – j1.402)  |  = 0.254

- X(6)

- Expected                 =   -4.0 – j4.0

- Obtained                 =   (fc.00 + jfc.00)H = -4 - j4

- Absolute  Error       =   0

- X(7)

- Expected                 =  -4 – j9.656

- Obtained                 =  (fc.00 + jf6.60)H = -4 – j9.625

- Absolute  Error       =  | (-4 – j9.656 ) – (-4 – j9.625) | = 0.031

Modifying the fft circuit to compute inverse fft

- Inverse fft can be computed using Data Swapping technique, as shown below:

- The real part of FFT input is fed as imaginary input to the circuit and the imaginary part of FFT is fed as real input to the circuit.

- The real portion of the output gives imaginary part of expected time domain sequence scaled by N  and  the imaginary part of the output gives the real part of the time domain sequence scaled by N. To get the desired outputs, we divide them by N.

- So, in addition to the existing hardware circuitry, extra circuitry will be required to scale by factor of (1/N) to get the desired time domain inverse FFT sequence.

Design Details:

- The entire FFT computation has been divided into three stages as follows:

- Stage 2 and Stage 3 are used for intermediate variable computations in the code.

- But the overall FFT computation can be broadly classified into stages 0, 1 and 4.

- If the RST_N signal is low, the value 0 is written in the  registers i0_real, i0_imag, i1_real,i1_imag, …… , i7_real, i7_imag.

- The output wires out0_real,out0_imag, out1_real,out1_imag,…..out7_real,out7_imag are continuously driven by the data present in the registers as shown below.

- In the above block, although only two registers are shown, there are total of 16 registers ( 8 for the real part and 8 for the imaginary part ) and each register holds 16 bit data.

- Storing Inputs in registers:

- The circuit stores inputs in registers when write signal is high.

- When write signal is high,  the registers i0_real ,i0_imag, i1_real,i1_imag,… ,i7_real, i7_imag will store the value held by the continuously driven wires in0_real,in0_imag,in1_real,in1_imag…..,in7_real,in7_imag respectively as shown below:

- Here as well only two signals in_real and in_imag are shown for brevity of block diagram.

- But there are 16 wires ( 8 for real and 8 for imaginary parts) that are continuously driven.

- Computation commences when start signal is high.

- The value stored by 2-bit register variable  named stage indicates the stage of FFT computation.

- Stage 0 computations:

- The contents in i0_real, i0_imag,…….,i7_real,i7_imag act as inputs for stage 0 computations.

- The registers g_real[0], g_imag[0], g_real[1],g_imag[1]…..,g_real[7],g_imag[7] store the results of stage 0.

- Once again for brevity, not all the registers of stage 0 are shown.

- There are 16 registers. 8 registers hold the real part of stage 0 results and 8 registers hold the imaginary part of stage 0 results .

- Stage 0 computations is based on the formula obtained from the butterfly diagram.

- Once stage 0 computations are done, the content of stage register is incremented by 1 to go to the next stage.

- Stage 1 computations

- For stage 1 computations, the contents of the registers g_real[0], g_imag[0], g_real[1],g_imag[1]…..,g_real[7],g_imag[7] will be the inputs and the results are stored in the registers

- The computations are based on the equations obtained from the butterfly diagram.

- Stage 2 computations

- In this stage , the registers temp0, temp1 ,temp2 and temp3 are populated as follows:

- temp0 =  h_real[5] + h_imag[5]

- temp1 =  h_imag[5] - h_real[5]

- temp2 =  h_imag[7] - h_real[7]

- temp3 = -(h_imag[7] + h_real[7])

- These calculations aid in the computation of the final result.

- Once the above calculations are done, contents of the stage register is incremented by 1 and we go to the next stage.

- Stage 3 computations

- In this stage, the contents of temp0, temp1, temp2, temp3 are multiplied with 0.707 ( hardcoded in binary in the code) .

- Then content of the stage register is incremented and we go to the final stage.

- Stage 4 computations:

- Here the contents of h_real[0],h_imag[0],h_real[1],h_imag[1]…..,h_real[7].h_imag[7] act as inputs and the final output is stored in the registers i_real[0],i_imag[0],i_real[1],i_imag[1],….i_real[7],i_imag[7].

- The wires out0_real , out0_imag, out1_real, out1_imag,…….out7_real,out7_imag are continuously driven by the contents in these registers. Hence at any point of time, the output at that instant is present in these output wires.

Architectural Trade-offs

- In this FFT implementation, focus is low latency.

- Hence the inbuilt multiplication function has been used using the ‘ * ‘ symbol to get faster multiplication results that are required at the intermediate stages.

- This comes at the cost of more area and power consumption.

- As an addition, pipelining can be implemented if we want to improve throughput.

References

- “DSP Tricks: Computing inverse FFTs using the forward FFT” – embedded,com